Creating your First PowerShell Script using

PowerShell Core SOP

Table of Contents

1. Creating your First PowerShell Script using PowerShell Core	3

1.1 Description	3

1.2 Architecture Diagram	3

1.3 Lab Steps	4

1.4 Troubleshooting	5

1.5 Supporting References	5

# Creating your First PowerShell Script using PowerShell Core

## Description

In this lab exercise, you will create a PowerShell script in a Virtual Machine that outputs process information to a file. You will have to use the appropriate PowerShell commands to filter the processes according to some parameters.

## Architecture Diagram

The diagram below displays a visual representation of the application architecture:

## Lab Steps

Follow the steps outlined below to achieve the objective of this lab exercise:

- Create a Linux Virtual Machine:

  - Go to the Azure Portal, click on Create a resource > Ubuntu Server 18.04 LTS b. Provide below details:

    - Resource Group: Your Resource Group name

    - Virtual Machine Name: <any-unique-name>

    - Region: Any (In case of any error, go as per the lab policy)

    - Image: Ubuntu Server 18.04 LTS-Gen1

    - Size: Standard_B1s

    - Authentication Type: Password

    - Give a username and password.

    - Leave other field as is and go to Management Tab.

    - Select Boot Diagnostics to Off

    - Leave other fields with their default values and click on Review+Create > Create.

- Install PowerShell:

  - Connect to your Linux Virtual Machine with the following command:

ssh username@<publicIPofVM>

  - Install PowerShell module in Linux VM:

sudo snap install powershell --classic

  - Once the installation is complete, launch the PowerShell prompt:

pwsh

- Create the PowerShell Script:

  - Collate all the commands which you want to put in your script.

  - The following command will give all the processes on your machine and store it in a variable:

$ps = Get-Process

  - Use pipelines to pass the output of Get-Process and only take the WorkingSet and ProcessName object type.

$ps = Get-Process | Select-Object WorkingSet,ProcessName

  - Use pipelines again to pass the output to Format-Table:

$ps = Get-Process | Select-Object WorkingSet,ProcessName | FormatTable

  - Use Pipeline last time, to store all this output in a csv file:

$ps = Get-Process | Select-Object WorkingSet,ProcessName | Format-

Table | Out-File processinfo.csv

  - Create a file by using the following command:

vi processinfo.ps1

  - Put the code defined in step 3e in this file and then save the file by entering: wq!

- Run the script:

  - Make sure that you are in PowerShell prompt.

  - Run the PowerShell script by typing ./processinfo.ps1 from the same directory as the file.

  - A processinfo.csv will be generated in the same directory.

  - Open the file using the following command:

Get-Content ./processinfo.csv

- Clean up resources:

  - When you are done with your lab exercise, delete all the resources created by you.

## Troubleshooting

| S. No | Problem | Solution |
| --- | --- | --- |
| 1 | Not able to select desired Region | Go as per the Lab Policy, look for any error and suggested options to be followed. |
| 2 | Permission Denied | Use the command with sudo privileges. |

## Supporting References

Refer the below links for additional information:

- https://docs.microsoft.com/en-us/powershell/scripting/install/installing-powershell-core-onlinux?view=powershell-7

- https://4sysops.com/archives/creating-powershell-core-scripts-on-linux/